IDT80KSW0001 PPS / IDT CPS-16/8 Evaluation Board

User Guide

Author: Ryan Morley

Date: May 4, 2007

Revision:  2.0

Revision History

| Revision | Date | Author | Comments |
| --- | --- | --- | --- |
| 0.5 | September 6, 2006 | Ryan Morley | Initial Draft. |
| 1.5 | April 5, 2007 | Ryan Morley | Updated for board revision 1.5 |
| 2.0 | May 4, 2007 | Ryan Morley | Updated for board revision 2.0 (19x19 mm package) |
| 2.25 | May 24, 2007 | Ryan Morley | Updated to include I2C EEPROM addressing issues. |

Table of Contents

Introduction	2

ATX Power Supply Input (1)	4

Vdd3 Power Supply(2)	4

1.2V Power Supply (Vdd, Vdda, Vdds) (3)	4

I2C Input Headers (4)	4

PPS SPD Switches (5)	4

Infiniband Connectors(6)	4

PPS I2C Address Switches (7)	5

JTAG Inputs (8)	5

Reference Clock (9)	5

AMC Connectors(10)	5

PPS Reset Pushbutton (11)	6

PPS (12)	6

SRIO SMA Connectors (13)	6

PPS Logic Signal Headers (14)	6

PCM Jumper (15)	6

# Introduction

This document is intended to provide an overview of the IDT80KSW0001 PPS/ IDT CPS-16/8 Evaluation Board Rev 2.0.  The information in this document applies to PPS/CPS Evaluation Board Rev 2.0, and is not necessarily compatible with PPS Evaluation Board Rev 1.0 or 1.5.

PPS/CPS Evaluation Board Layout

| PPS/CPS Evaluation Board Sections |  |
| --- | --- |
| 1 | ATX power supply input |
| 2 | Vdd3 Power Supply |
| 3 | 1.2 V power supply (Vdd, Vdda, and Vdds) |
| 4 | I2C input headers |
| 5 | PPS/CPS SPD Switches |
| 6 | Infiniband connector |
| 7 | PPS/CPS I2C Address Switches |
| 8 | JTAG Inputs |
| 9 | Reference Clock |
| 10 | AMC connectors |
| 11 | PPS/CPS Reset Pushbutton |
| 12 | PPS/CPS |
| 13 | SRIO SMA connectors |
| 14 | PPS/CPS Logic/Control Signal Headers |
| 15 | CPS EEPROM |
| 16 | SRIO Logic analyzer connections |

# ATX Power Supply Input (1)

The PPS/CPS Evaluation Board is designed for use with a standard 20-pin ATX power supply.   The evaluation board is powered by the 12V source on the ATX supply.  Other Voltages from the ATX supply are not used by the evaluation board; however, these voltages can be accessed through the 0.100” headers included on the board.

The ATX supply should be able to provide approximately 150 Watts at 12V.  This will allow for power supply margin under full load (2 AMC modules) condition.

# Vdd3 Power Supply(2)

Component “Vdd3” is a 3.3V linear supply.  Vdd3 is used to provide 3.3V power to the PPS/CPS, as well as to provide 3.3V management power to the AMC connectors.  The output of Vdd3 is connected to the evaluation board by fuse F_Vdd3.  Vdd3 can be powered-down by installing jumper Vdd3_Off.

# 1.2V Power Supply (Vdd, Vdda, Vdds) (3)

The Voltage regulator U1v2 is used to provide 1.2V to the PPS supplies.  U1v2 powers Vdd, Vdds, and Vdda.  However, each of these Voltage domains are individually fused (F_Vddx) and distributed to the PPS through separate planes.

The output Voltage level of U1v2 is controlled by Rtrim.  The default value of Rtrim, 402 Ohms, should provide an output of 1.2V.  This level can be adjusted by changing the value of Rtrim.

U1v2 can be powered down by installing jumper Vdd_off.

# I2C Input Headers (4)

The PPS/CPS I2C bus is connected to headers SDA_in and SCL_in.  10 pin header aard_iic is provided for connection to the Totalphase Aardvark I2C/SPI adapter.

# PPS/CPS SPD Switches (5)

The PPS/CPS SPD pins are connected to DIP switch SW_SPD, and can be set to either 0 or 1.  This controls the reset bit rate of the PPS/CPS SRIO ports.

# Infiniband Connector(6)

One Infiniband 4x connector is provided on the evaluation board.  This is to allow for the connection of multiple evaluation boards.  Please refer to Appendix A:  High Speed Lane Mapping for lane mapping of the Infiniband connector.

# PPS/CPS I2C Address Switches (7)

SW_IIC is a 12-pos DIP switch for setting the PPS/CPS I2C address.  Switch ADS selects 10-bit or 7-bit address mode.  Switches ID9-ID0 set the PPS I2C address.

For CPS applications, switch Master_Mode enables CPS I2C master mode.

# JTAG Inputs (8)

Connector JTAG provides access to the PPS/CPS JTAG pins.  Connector JTAG is a 10 pin, 100 mil IDC header, which can be used with the Corelis JTAG controller.

# Reference Clock (9)

The PPS/CPS is designed to operate with an AC coupled reference clock, at a frequency of 156.250 MHz.  This can be implemented on the Evaluation Board using either an external clock source or the onboard crystal oscillator.

To use the onboard crystal oscillator, set Clk+ and Clk- jumpers to ‘On_Board’ setting.

To use an external clock source, set Clk+ and Clk- jumpers to ‘External’ setting, and connect a clock source to CLK_IN+ and CLK_IN- SMAs.  10pF shunt capacitors Cext_clk+ and Cext_clk- can be installed to slow the edge rate of the external clock signal when using equipment with very fast rise times.  These are not installed in the default board setup.

The evaluation board is designed for use with PECL clock oscillators.  For use of LVDS oscillators, remove Rdn_clk+/- and Rup_clk+/-, and replace 50 Ohm Rs_clk+/- with 0-Ohm resistors.

# AMC Connectors(10)

Two AMC B+ connectors are provided on the evaluation board for use with AMC modules.  These connectors do not implement full AMC functionality.

PS1# and PS0# pins are tied to proper levels.  However, they are not used to control ENABLE#.

ENABLE# is held low by jumper AMCx_en.  To disable the AMC module, remove jumper AMCx_en.

GA0-2 pins are floating (NC).

AMC JTAG and I2C pins are floating; header access is provided to these pins, if needed.

CLK1-3 are not implemented.

Four ports of pass-through connectivity are provided as a direct high-speed connection between AMC1 and AMC2.

Please refer to Appendix A:  High Speed Lane Mapping for lane mapping of the AMC connectors.

# PPS/CPS Reset Pushbutton (11)

Pushbutton PPS_RST is used to hard-reset the PPS.  Jumper J_Rst must be installed.  Direct access to the PPS RST pin is available at the ‘Local’ pin of header J_Rst.

# PPS/CPS (12)

The PPS/CPS Evaluation Board is designed for either solder-down or socketed use of the PPS/CPS.  For socketed applications, use Ironwood Electronics C7439 GHz BGA socket.

# SRIO SMA Connectors (13)

Lanes 0 – 3 of the PPS/CPS can be accessed through SMA connectors.  AC-coupling capacitors are placed on the receiver paths.

If connecting the SMA outputs to test equipment (50 Ohm – Gnd termination), SMA – SMA DC – Blocks should be used.

# PPS/CPS Logic/Control Signal Headers (14)

Access to the PPS/CPS logic and test pins is available at headers located near the PPS/CPS.

# CPS EEPROM (15)

An I2C EEPROM is included on the Evaluation Board for use in CPS Master Mode applications.  The EEPROM I2C address is 1010(ID2)(ID1)(ID0), where ID2:0 are the three LSBs of the CPS I2C address.

To avoid I2C bus contention, PPS/CPS signals ID6:3 should not be set to 1010.

On this board, logic level 1 for ID signals = 1.2 V.  However, logic level 1 required for the EEPROM = 3.3V.  Because of this, ID2:0 should be set to 3b000 when using the EEPROM.  This should be corrected on future revisions of the evaluation board.

# SRIO LA Connections (16)

Compression style logic analyzer connections are provided for functional probing SRIO traffic.  Please refer to Appendix A:  High Speed Lane Mapping for signal availability on the SRIO LA connections.

# Appendix A:  High Speed Lane Mapping

The following table outlines the high speed lane mapping of the PPS/CPS Evaluation Board Rev 2.0 for both PPS and CPS applications.

| PPS/CPS Lane number |  | Connections |
| --- | --- | --- |
| 0 | Quad 0 | SMA |
| 1 |  | SMA |
| 2 |  | SMA |
| 3 |  | SMA |
| 4 | Quad 1 | AMC1 Port 4 |
| 5 |  | AMC1 Port 5 |
| 6 |  | AMC1 Port 6 |
| 7 |  | AMC1 Port 7 |
| 8 | Quad 2 | AMC2 Port 4 |
| 9 |  | AMC2 Port 5 |
| 10 |  | AMC2 Port 6 |
| 11 |  | AMC2 Port 7 |
| 12 | Quad 3 (CPS Only) | Infiniband 1, Lane 0 |
| 13 |  | Infiniband 1, Lane 1 |
| 14 |  | Infiniband 1, Lane 2 |
| 15 |  | Infiniband 1, Lane 3 |
|  | Pass Through 0 | AMC1 Port 12 to AMC2 Port 12 |
|  | Pass Through 1 | AMC1 Port 13 to AMC2 Port 13 |
|  | Pass Through 2 | AMC1 Port 14 to AMC2 Port 14 |
|  | Pass Through 3 | AMC1 Port 15 to AMC2 Port 15 |